# Magnetometer Time Series Simulator

Version: 1.0.0 | Author: Strider Settgast with Deepseek

This module provides a high-fidelity simulator for generating synthetic magnetometer data from dipole sources. It is specifically designed for generating training data for neural networks focused on magnetic source localization.

## 1. Enumerations

### SourceType

Types of magnetic sources that can be simulated.

- DIPOLE ("dipole"): Standard magnetic dipole.

- MONOPOLE ("monopole"): Not physically real but useful for testing/baselines.

- QUADRUPOLE ("quadrupole"): Planned for future extension.

### NoiseModel

Available noise models for sensor simulation.

- GAUSSIAN ("gaussian"): Standard normally distributed noise.

- UNIFORM ("uniform"): Uniformly distributed noise matching the variance of the Gaussian model.

- MIXED ("mixed"): A combination of Gaussian noise and extreme outliers (designed for robust testing).

## 2. Configuration: SimulatorConfig

A dataclass used to configure the physics and generation settings of the simulator.

### Parameters / Attributes

| Parameter | Type | Default Value | Description |
| --- | --- | --- | --- |
| sensor_positions | np.ndarray | Required | Array of shape (n_sensors, 3) representing sensor coordinates (X, Y, Z) in meters. |
| magnetic_constant | float | 1e-7 | in SI units. Set to 1.0 for normalized output. |
| default_source_bounds | Tuple[float, float] | (-5.0, 5.0) | [min, max] boundaries for the random source position cube. |
| default_moment_range | Tuple[float, float] | (0.5, 2.0) | [min, max] boundaries for the magnetic moment magnitude. |
| noise_std | float | 0.01 | Standard deviation for the Gaussian noise model. |
| noise_outlier_fraction | float | 0.01 | Fraction of data points that will be generated as outliers when using the MIXED noise model. |
| random_seed | Optional[int] | 42 | Seed for numpy's random number generator to ensure reproducibility. |
| normalize_outputs | bool | False | If True, normalizes the target output positions to a [-1, 1] range based on output_bounds. |
| output_bounds | Optional[Tuple] | None | Bounds used for output normalization. Note: If left as None, it automatically defaults to the values set in default_source_bounds upon initialization. |

Computed Attributes (Generated after initialization):

- n_sensors (int): Automatically calculated from the length of sensor_positions.

## 3. Main Class: TimeSeriesSimulator

The core class that generates realistic magnetic field readings from moving or static dipole sources.

### Initialization

sim = TimeSeriesSimulator(sensor_positions: np.ndarray, config: Optional[SimulatorConfig] = None)

Instance Attributes (Available after initialization):

- sensor_positions (np.ndarray): Pre-computed array of sensor locations.

- n_sensors (int): Number of sensors.

- n_features (int): Total number of features generated per time step (n_sensors * 3, representing Bx, By, Bz per sensor).

- config (SimulatorConfig): The active configuration object.

- input_mean / input_std / output_mean / output_std: Placeholders (initialized to None) intended for online normalization statistics.

### Core Physics & Generation Methods

#### dipole_field(sensor_pos, source_pos, magnetic_moment)

Calculates the theoretical magnetic field from a dipole source at a single specific sensor location.

- Parameters:

  - sensor_pos (np.ndarray): (3,) array of sensor position [x, y, z].

  - source_pos (np.ndarray): (3,) array of source position [x, y, z].

  - magnetic_moment (np.ndarray): (3,) array of magnetic moment vector [mx, my, mz].

- Returns: np.ndarray of shape (3,) representing the magnetic field [Bx, By, Bz].

#### compute_fields(source_pos, magnetic_moment)

Computes the clean, noiseless magnetic fields across all sensors simultaneously.

- Parameters:

  - source_pos (np.ndarray): (3,) array.

  - magnetic_moment (np.ndarray): (3,) array.

- Returns: np.ndarray of shape (n_features,). Flattened array of Bx, By, Bz for all sensors.

#### generate_source()

Generates a random magnetic source based on the bounding boxes defined in the configuration.

- Returns: Tuple[np.ndarray, np.ndarray] containing (source_position, magnetic_moment). Position is uniformly distributed within bounds; moment direction is a uniform sphere distribution.

### Data Generation Pipelines

#### generate_sample(source_pos=None, magnetic_moment=None, add_noise=True, noise_model=NoiseModel.GAUSSIAN)

Generates a single paired training sample (features and target).

- Parameters:

  - source_pos / magnetic_moment (Optional[np.ndarray]): If None, randomly generated.

  - add_noise (bool): Toggle noise application. Default: True.

  - noise_model (NoiseModel): Which noise distribution to use.

- Returns: Tuple[np.ndarray, np.ndarray] containing (features, target). features is shape (n_features,), target is shape (3,).

#### generate_batch(n_samples, add_noise=True, noise_model=NoiseModel.GAUSSIAN)

Generates a dataset of independent, static samples.

- Parameters:

  - n_samples (int): Number of independent samples to create.

- Returns: Tuple[np.ndarray, np.ndarray] containing features matrix X of shape (n_samples, n_features) and targets matrix y of shape (n_samples, 3).

#### generate_time_series(n_timesteps, trajectory_func=None, moment_func=None, dt=0.01, add_noise=True, noise_model=NoiseModel.GAUSSIAN)

Generates sequential time-series data mimicking a moving source.

- Parameters:

  - n_timesteps (int): Number of sequential data points to generate.

  - trajectory_func (callable, optional): A function taking time t and returning a (3,) position array. If None, defaults to a random walk constrained by the source bounds.

  - moment_func (callable, optional): A function taking time t and returning a (3,) moment array. If None, defaults to a constant random moment.

  - dt (float): Time step delta in seconds. Default 0.01.

- Returns: Tuple[np.ndarray, np.ndarray] containing time series X (n_timesteps, n_features) and y (n_timesteps, 3).

### Utility, Analysis, and I/O Methods

#### add_noise(fields, noise_model=NoiseModel.GAUSSIAN)

Applies noise to a clean field array based on the specified noise model enum.

- Returns: np.ndarray of noisy field readings.

#### compute_snr(clean_fields, noisy_fields)

Calculates the Signal-to-Noise Ratio (SNR) for the generated data.

- Returns: float representing the SNR in decibels (dB). Returns inf if noise is identically zero.

#### get_data_statistics(X, y)

Computes comprehensive statistical metadata about a generated dataset.

- Returns: Dict containing mean, std, min, max, has_nan, and has_inf for both the input (X) and output (y) arrays.

#### visualize_sensor_array(ax=None)

Plots a 3D visualization of the sensor setup. Requires matplotlib.

- Parameters: ax: An optional matplotlib 3D axis. If None, creates a new figure and axis.

- Returns: The matplotlib axis object (ax).

#### save_dataset(X, y, filepath)

Saves generated features, targets, sensor positions, and configuration to a compressed numpy array file.

- Parameters: filepath (str): File destination (should end in .npz).

#### @classmethod load_dataset(filepath)

A class method to easily load datasets generated by save_dataset.

- Returns: Dict containing keys: 'X', 'y', 'sensor_positions', and 'config'.